# ECA5604- 5g/6g communication

Slot A

B.vennela

192412604

Experiment 2: Implementing TDMA Slot Allocation in GSM.

AIM: To implement TDMA (Time Division Multiple Access) slot allocation in GSM using

MATLAB and analyze system performance in terms of time slot duration, frame utilization,

and number of allocated slots.

Objective 1: To calculate the Time Slot Duration in a GSM TDMA frame by dividing the total frame

time into equal time slots. This helps in understanding the time structure of GSM

code:

clc; clear; close all;

FrameTime = 4.615; Slots = 8;

SlotDuration = FrameTime/Slots;

t = 0:SlotDuration:FrameTime;

figure

subplot(2,1,1)

stem(t,ones(size(t)),'filled')

title('TDMA Time Slot Structure')

xlabel('Time (ms)')

ylabel('Slot Level')

grid on

subplot(2,1,2)

plot(t,ones(size(t)),'-o','LineWidth',1.5)

title('Continuous View of Slot Allocation')

xlabel('Time (ms)')

ylabel('Slot Presence')

grid on

output:

Result:

• The graph shows discrete time slots arranged in a GSM TDMA frame structure with

equal spacing.

• It confirms fixed time division where each slot is allocated a specific time interval

without overlap.

Objective 2: To determine the Frame Utilization (%) based on the number of allocated time slots

compared to total available slots. This evaluates the efficiency of TDMA resource usage.

Code:

clc; clear; close all;

TotalSlots = 8;

AllocatedSlots = 6;

FreeSlots = TotalSlots - AllocatedSlots;

Utilization = (AllocatedSlots/TotalSlots)*100;

disp('Frame Utilization (%)')

disp(Utilization)

figure

% Graph 1: Simple Line Plot (SAFE - no bar function)

subplot(2,1,1)

plot([1 2],[AllocatedSlots FreeSlots],'-o','LineWidth',2)

title('GSM Frame Utilization (Allocated vs Free)')

xlabel('Slot Type Index (1=Allocated, 2=Free)')

ylabel('Number of Slots')

grid on

output:

Result:

The plot shows that most of the TDMA frame is allocated to active users, indicating high

resource usage.

The pie chart clearly represents that 75% of slots are utilized while 25% remain free.

Objective 3: To compute the Number of Allocated Time Slots for multiple users in the GSM system.

Code:

clc; clear; close all;

Users = 1:8; % Total users

TotalSlots = 5; % Available slots

Alloc = zeros(1,8); % Initialize allocation

Alloc(1:TotalSlots) = 1; % Assign slots to first users

Blocked = 1 - Alloc; % Remaining users blocked

disp('User Wise Slot Allocation')

disp(table(Users',Alloc','VariableNames',{'User','Status(1=Allocated,0=Blocked)'}))

figure

% Graph 1: Allocation per user

subplot(2,1,1)

stem(Users,Alloc,'filled','LineWidth',2)

title('TDMA Slot Allocation per User')

xlabel('User Index')

ylabel('Slot Status (1=Allocated, 0=Blocked)')

ylim([-0.2 1.2])

grid on

% Graph 2: System view (safe alternative to bar)

subplot(2,1,2)

plot(Users,Alloc,'-o','LineWidth',2)

hold on

plot(Users,Blocked,'-s','LineWidth',2)

title('TDMA Slot Utilization Analysis')

xlabel('User Index')

ylabel('Status')

legend('Allocated','Blocked')

grid on

output:

Result:

1. The time slot duration in the GSM TDMA frame was successfully calculated, confirming

equal division of the frame into fixed time intervals.

2. The frame utilization was effectively analyzed, showing the proportion of allocated and

free slots in the TDMA system.

3. The slot allocation per user was successfully implemented, demonstrating that limited

slots are assigned to users while excess users are blocked due to resource constraints.